Track 1 — Parts List & Procurement

Dedicated hardware to acquire and validate the OBD-II + TPMS + GPS data path

Car Automation IoT Internship   •   For budget sign-off

# 1. What We Are Buying For

This list covers everything needed to acquire and test the Track 1 vehicle-data path end to end: reading OBD-II parameters in-house, reading our own tyre sensors, tagging everything with GPS, and streaming it to the backend. It is built around the Vehicle Data Acquisition Standard (v1.0.0).

This is a fresh, dedicated set of hardware for Track 1. All core boards (ESP32, GPS, CAN) are purchased new rather than reused, so Track 1 has its own standalone rig that is not shared with or cannibalised by other work. A two-board layout is used: one board for OBD + TPMS acquisition, one for GPS + CAN.

The data path is three separate subsystems, not one feed:

- OBD adapter → ESP32 → drivetrain, engine health and fuel data (RPM, speed, coolant, load, throttle, intake, fuel level, DTCs).

- 433 MHz receiver + our own tyre sensors → the 16-byte TPMS packet (tyre pressure and temperature).

- GPS module → location, timestamp and satellite count.

In-house requirement: the OBD adapter is wired into our own hardware and decoded in our own code — the adapter’s phone app is not used. This is why the wired ELM327 is the primary pick over the app-based Bluetooth/WiFi units, even though those log faster: the app path is not permitted.

# 2. Core Hardware

The controller boards and core modules, purchased fresh for a dedicated Track 1 rig.

| Item | Qty | Unit ₹ | Total ₹ | Role / Notes |
| --- | --- | --- | --- | --- |
| ESP32 development board | 2 | 500–650 ea | 1,000–1,300 | A spare board protects against a bricked/damaged unit during hardware work. |
| NEO-6M GPS module | 1 | 350–500 | 350–500 | With ceramic patch antenna. Provides location, UTC timestamp and satellite count for the 32-byte packet. |
| MCP2515 CAN module | 2 | 150–250 ea | 300–500 | CAN interface (TJA1050 transceiver). One as the CAN/backup OBD read-path (reading PIDs directly off CAN), one spare / for the second board. Kept as a fallback if the ELM327 proves unreliable. |

# 3. OBD-II Acquisition

| Item | Qty | Approx ₹ | Notes |
| --- | --- | --- | --- |
| ELM327 OBD-II adapter | 1 | 1,300–2,000 | PRIMARY pick. Wired, all OBD2 protocols incl. ISO 15765 (CAN). See §5 on the wired-connection choice (USB vs UART pins) — decide before ordering. Buy a decent unit, not the cheapest clone (clone chips are the usual cause of “not working properly”). |
| OBD-II extension cable | 1 | 400–500 | 16-pin male-to-female, ~1–1.5 m. Lets us plug in comfortably and work without contorting under the dash while testing. |
| USB-to-TTL / connection interface | 1 | 200–400 | Only if using a USB adapter with the ESP32 — the ESP32 is not a USB host, so a USB ELM327 does not plug straight in. Not needed if a UART-pin ELM327 or a laptop is used. See §5. |

# 4. TPMS Acquisition & Wiring

## 4.1 TPMS (separate 433 MHz path)

| Item | Qty | Approx ₹ | Notes |
| --- | --- | --- | --- |
| Aftermarket TPMS sensors (set of 4) | 1 set | 2,000–2,500 | External valve-cap type, 433 MHz. Sold as kits with a display we don’t need. IMPORTANT: packet format varies by brand — confirm the brand before ordering, it determines the decode work. |
| 433 MHz receiver module (CC1101) | 1 | 150–300 | SPI transceiver, reads the TPMS broadcasts for the ESP32 to decode. Preferred over a plain superheterodyne receiver (SRX882) because it is tunable and better suited to decoding varied TPMS packets. |

## 4.2 Wiring & misc

| Item | Qty | Approx ₹ | Notes |
| --- | --- | --- | --- |
| Jumper wires (M-M, M-F) | 1 set | 80 | Connections between modules and the ESP32 boards. |
| Small perfboard / breadboard | 1 | 60 | Mounting the receiver and power rails cleanly. |
| Breadboard power / USB cables | 1 set | 100 | Powering and flashing both boards. |

# 5. Decision Before Ordering: How the Adapter Connects

This must be decided before buying the ELM327, because it changes which unit to order. A USB ELM327 has a USB plug, but the ESP32 is not a USB host — a USB dongle does not plug straight into the ESP32. There are three viable setups:

| Setup | What it means | What to buy |
| --- | --- | --- |
| 1. USB adapter → laptop | Simplest to start. Laptop runs our decode code and sends data to the backend over its own internet. Good for the first real-car test — fastest way to prove the read path works. Downside: a laptop sits in the car during testing. | USB ELM327 + laptop |
| 2. UART-pin adapter → ESP32 | Fully in-house, no laptop. Requires an ELM327 board that exposes raw TX/RX serial pins (not a sealed USB dongle). Cleanest final setup — the ESP32 reads it directly over UART. | UART-pin ELM327 |
| 3. USB adapter → USB-host module → ESP32 | Works but adds a USB-host interface and complexity. Only if a USB unit must be used with the ESP32. | USB ELM327 + USB-host module |

Recommended approach: start with Setup 1 (USB adapter + laptop) to prove the car actually talks and the decode logic is correct — this is the fastest way to get real data and de-risk the whole track. Then move to Setup 2 (UART-pin adapter → ESP32) for the final in-house version once the decode is proven.

# 6. Alternatives Considered

## 6.1 OBD adapter alternatives

| Option | Status | Approx ₹ | Trade-off |
| --- | --- | --- | --- |
| Wired USB ELM327 | PRIMARY | 1,300–2,000 | Wires directly into our hardware; simple UART/serial-style integration; fits the in-house requirement. Basic buffer can lag under fast polling — mitigated by our 1 Hz poll rate. |
| Vgate iCar Pro (WiFi) | Alternative | 2,800–4,200 | Phone-free wireless link, higher throughput than cheap clones. But it needs WiFi socket handling in firmware and leans toward app-based logging — less clean for direct in-house ESP32 wiring. |
| OBDLink MX+ (Bluetooth) | Alternative | 6,500–8,000 | Best data quality — STN chipset, large CAN buffer, up to 4× sample rate. But it is built for app-based logging (the path ruled out) and needs a Bluetooth stack in firmware. Overkill for our 1 Hz in-house use. |

## 6.2 433 MHz receiver alternatives

| Option | Status | Approx ₹ | Trade-off |
| --- | --- | --- | --- |
| CC1101 module | PRIMARY | 150–300 | Tunable SPI transceiver, better for decoding varied TPMS packet formats. The safer pick given TPMS decode is the hard part. |
| SRX882 superheterodyne receiver | Alternative | 100–150 | Cheaper and simpler, but cruder — fixed, less capable for decoding non-trivial TPMS packets. Fine only if the chosen sensor packet turns out to be simple. |

## 6.3 OBD read method — adapter vs direct CAN

ELM327 adapter (chosen): the adapter handles the low-level protocol and hands us clean hex; we decode PIDs in our own code. This is the requested approach and the fastest to working data.

Direct CAN via MCP2515 (backup): we are also buying the MCP2515 modules, so we can read OBD PIDs straight off the CAN bus without any adapter if needed — arguably the most in-house option of all. Kept as a fallback if the ELM327 proves unreliable.

# 7. Cost Summary

| Section | Approx ₹ |
| --- | --- |
| Core hardware (2× ESP32, GPS, 2× CAN) | 1,650 – 2,300 |
| OBD acquisition (adapter, cable, interface) | 1,900 – 2,900 |
| TPMS acquisition (sensors + CC1101) | 2,150 – 2,800 |
| Wiring & misc | ~240 |
| Track 1 total | ≈ 5,900 – 8,200 |

# 8. Sourcing

Parts can be bought locally or online:

- Local: Mercy Electronics (Tambaram), Swastik Electronics (near VIT).

- Online: Robu.in, Robocraze, ThinkRobotics, Quartz Components, and Amazon.in for anything not stocked locally.